WFH07 Bluetooth Communication Protocol

Version <A4.0>

Version history

| Version | Prepared by | Date of Preparation | Reviewed by | Date | Status |
| --- | --- | --- | --- | --- | --- |
| A1.0 | Yuhui | 2024.03.27 | Tony | 2024.03.29 |  |
| A3.0 | Yuhui | 2024.11.04 | Tony | 2024.11.11 | Add flashing and remote control |
| A4.0 | Yuhui | 2024.12.24 | Tony | 2024.12.24 | Modify flashing and remote control |

Contents

1. Electronic Hardware Diagram	2

2. Electronic hardware specifications	4

3. Bluetooth working diagram	5

4. Bluetooth technical parameters 	 5

5. Bluetooth Service Description	6

# 1. Electronic Hardware Diagram

Figure 1

Figure 2

| Module | Function | Remarks |
| --- | --- | --- |
| Bluetooth Low Energy | Realize data communication and exchange between smart housing and mobile APP according to Bluetooth protocol. | Version: BLE5.0 |
| Power module | Including the power supply voltage regulator control module, lithium battery charge and discharge management module, and lithium battery voltage detection module. Manage the charge and discharge, power stability and voltage detection of lithium battery. | Please refer to Section 5. 1 for details. |
| Automatic vacuum pumping module | Including Extraction motor control circuit and Air pressure detection circuit. By controlling the start and stop of the motor, the air inside the waterproof casing is extracted to form a negative air pressure state, so as to simulate the sealing condition of the casing. During the pumping process, the current air pressure change inside the waterproof case is continuously detected, and data analysis is used to determine whether there is a risk of water leakage. | To predict water leakage by calculating air pressure, refer to Section 5.4 for details. |
| Key module | Including the combination button and shutter button, the currently pressed button information is detected, processed by the Bluetooth module inside the product, and then the information is sent to the mobile phone through Bluetooth wireless transmission in the form of a specific code | The Bluetooth service can obtain data for the key, please refer to Section 5.3 for details. |
| Water depth and temperature detection module | The water pressure and water temperature of the current environment are detected by the water pressure sensor installed in the casing, and the data is wirelessly transmitted to the mobile phone through Bluetooth. | The Bluetooth service can obtain data for the sensor, see section 5.4 for details. |
| LED display module | The current Bluetooth status, charging status and sealing status of the casing can be indicated by the different states displayed though the LED indicator. | Red light on: charging Green light on: charging complete Blue light on: Bluetooth is connected Blue light flashing: Bluetooth is waiting to connect. |

Table 1

Remarks：

( 1) The data sent from the casing to the mobile phone via Bluetooth wireless

transmission：

A. Battery level data

B. Key detection data

C. Data of water pressure and temperature detected by water pressure sensor

Data from the barometric sensor

D. Data of device parameters (e.g., manufacturer, firmware version, etc.)

E. Air extraction cover signal

(2)  Data sent from mobile phone to the casing through Bluetooth wireless

transmission:

A 、Data of the air extraction motor control command issued from the mobile APP

B、 Data of the solenoid valve control command issued from the mobile APP

C、Data of the Trigger flash control command issued from the mobile APP

D 、OTA firmware upgrade data

(3) How to use this product for automatic vacuum pumping

When the user needs to turn on vacuum detection, the following steps need to be taken:

1. Confirm opening the suction cover for motor exhaust

2. Open the solenoid valve to prevent the exhausted gas from entering the interior of the waterproof shell again

3. Turn on the motor and start pumping air

4. When the specified air pressure(Can use the internal air pressure value of the phone, Hardware can cancel the pressure sensor) is reached, stop pumping and turn off the motor

5. Confirm that the suction cover is closed, and the detection of small waterproof leaks is not accurate

6. Close the solenoid valve so that the waterproof shell can be easily opened when opening the suction cover. Close the solenoid valve in advance to save power

7. Check for more than 5 minutes and determine if there is any air or water leakage based on the air pressure

# 2. Electronic hardware specifications

| Bluetooth | Protocol | Transmitting power | Communication distance | Reception sensitivity | Voltage |
| --- | --- | --- | --- | --- | --- |
|  | BLE 5.0 | 4dBm | 100m | -95dBm | 1.8~3.6V |
| Battery | Battery type | Capacity | Voltage | Charge and discharge times | Charging time |
|  | Lithium polymer | 300mah | 3.7V | 500 times | Appro. 2H |
| Charging type | TYPE C 5V/ 1A |  |  |  |  |
| Run time | > 24H (estimated) |  |  |  |  |
| Standby time after shutdown | > 60D (estimated) |  |  |  |  |
| Electric pumping times | 100 times ( estimated) |  |  |  |  |
| Water depth detection | 0~ 1400Kpa Resolution 50 Kpa |  |  |  |  |
| Air pressure detection | 30 Kpa ~120 Kpa Resolution 5 Kpa |  |  |  |  |
| Times of button use | > 10000 times |  |  |  |  |

Table 2

# 3.  Bluetooth working diagram

Figure 2

# 4.  Bluetooth technical parameters

4. 1   Bluetooth Advertising Parameters

| Advertising parameters | Contents | Remarks |
| --- | --- | --- |
| Advertising_Type | Connectable Undirected Event Type |  |
| Advertising_Name | DIVE IT |  |
| Advertising_Data | MAC address, 6 bytes |  |
| Advertising_Address_Type | Static Device Address |  |
| Advertising interval | 200MS |  |
| Advertising duration | 180S |  |

Table 3

4.2  Bluetooth connection parameters

| Connection parameters | Contents | Remarks |
| --- | --- | --- |
| _ _ min conn interval | Can be specified | Current:25ms |
| _ _ max conn interval | Can be specified | Current:125ms |
| Slave Latency | Can be specified | Current: 0 |
| Supervision Timeout | Can be specified | Current: 4S |

Table 4

To modify these parameters, the iPhone must meet the following conditions:

① Max_conn_interval *  (Slave Latency + 1) <= 2s

② Min_conn_interval >= 20 ms

③ Min_conn_interval + 20 ms <= max_conn_interval

④ Slave Latency >= 4

⑤ Supervision Timeout <= 6s

⑥ max_conn_interval *  (Slave Latency + 1) * 3  < Supervision Timeout

# 5.  Bluetooth Service Description  (service)

5. 1  Battery Service

| Service type | Bluetooth SIG standard service |
| --- | --- |
| Service UUID | 0x180F |
| Battery level data Characteristic UUID | 0x2A19 |
| Battery level data permissions | Read and Notification |
| Battery data bytes | 1 byte |
| Data content | 0~ 100 indicates that the current battery level ranges from 0 to 100% |
| Example | 0x64 indicates that the current Bluetooth battery is 100% |

Table 5

5.2  Device information Service

| Service type | Bluetooth SIG standard service |
| --- | --- |
| Service UUID | 0x180A |
| UUID (Manufacturer Name Characteristic) | 0x2A29 |
| UUID (Software Revision Characteristic ) | 0x2A28 |
| UUID (Hardware Revision Characteristic) | 0x2A27 |
| UUID (Firmware Revision Characteristic) | 0x2A26 |
| UUID (Serial Number Characteristic) | 0x2A25 |
| Data read permission | Read Only |
| Example | Manufacturer Name : UMEING |

Table 6

5.3  Key service

| Service type | Bluetooth private service |  |  |  |
| --- | --- | --- | --- | --- |
| Service basis UUID | 0x23, 0xD1, 0xBC, 0xEA, 0x5F, 0x78, 0x23, 0x15, 0xDE, 0xEF, 0x12, 0x12, 0xxx, 0xxx ,0x00, 0x00, 0x00, 0x00 |  |  |  |
| Service UUID | 0x1523 |  |  |  |
| Key Characteristic UUID | 0x1524 |  |  |  |
| Key data permissions | Read and Notification |  |  |  |
| key data bytes | 1byte |  |  |  |
| Key value | Item |  | Short Press | Long press |
|  | Key 1 (right) |  | 0x10 |  |
|  | Key 2 (shutter) |  | 0x20(Power on) |  |
|  | Key 3 (up) |  | 0x30 |  |
|  | Key 4 (left) |  | 0x40 |  |
|  | Key 5 (ok) |  | 0x50 | Power off |
|  | Key 6 (down) |  | 0x61 | 0x60 |
|  | Key 7 (Z+) |  | 0x70 |  |
|  | Key 8 (Z-) |  | 0x80 |  |
| Example | Bluetooth notifies the phone that there is 0x10 data, which means Key 1 has been short-pressed. |  |  |  |
| Trigger flash LED Characteristic UUID | 0x1525 |  |  |  |
| Trigger flash LED data permission | Write |  |  |  |
| Trigger flash LED data byte | 1byte |  |  |  |
|  | 0x01(Any number) | Trigger flash once |  |  |
| Example | 0x01 indicates Trigger flash once |  |  |  |

Table 7

5.4 Water pressure and temperature sensor, barometric pressure sensor data reporting, vacuuming start indication, Air extraction cover signal service

| Service type | Bluetooth private service |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service basis UUID | 0x23, 0xD1, 0xBC, 0xEA, 0x5F, 0x78, 0x23, 0x15, 0xDE, 0xEF, 0x12, 0x12, 0xxx, 0xxx ,0x00, 0x00, 0x00, 0x00 |  |  |  |  |  |  |  |  |
| Service UUID | 0x1623 |  |  |  |  |  |  |  |  |
| Motor control characteristics UUID | 0x1624 |  |  |  |  |  |  |  |  |
| Start data permission | Write |  |  |  |  |  |  |  |  |
| Start data byte | 1 byte |  |  |  |  |  |  |  |  |
|  | 0x01 |  | Vacuuming start (non - 0 data) |  |  |  |  |  |  |
|  | 0x00 |  | Vacuuming Stop |  |  |  |  |  |  |
| Example | 0x01 indicate that vacuuming start |  |  |  |  |  |  |  |  |
| Water pressure sensor characteristics UUID | 0x1625 |  |  |  |  |  |  |  |  |
| Sensor data permission | Read and Notification |  |  |  |  |  |  |  |  |
| Sensor data bytes | 4 bytes (Little-Endian） |  |  |  |  |  |  |  |  |
| Sensor pressure data value: 32-bit unsigned integer (0.1mba) | 1st byte (MSB) |  |  |  | 2nd byte |  | 3rd byte |  | 4th byte (LSB) |
|  | (LSB) |  |  |  |  |  |  |  | (MSB) |
| Example | 0x83270000 indicates that the current water pressure is 1011.5mba (0x2783) |  |  |  |  |  |  |  |  |
| Water pressure sensor temperature characteristic UUID | 0x1626 |  |  |  |  |  |  |  |  |
| Sensor data permission | Read and Notification |  |  |  |  |  |  |  |  |
| Sensor data bytes | 4 bytes (Little-Endian） |  |  |  |  |  |  |  |  |
| Sensor water temperature data value: 32-bit unsigned integer (0.01℃) | 1st byte (MSB) |  |  | 2nd byte |  | 3rd byte |  | 4th byte (LSB) |  |
|  | (LSB) |  |  |  |  |  |  | (MSB) |  |
| Example | 0x580b0000 indicates that the current water temperature is 29.04 ℃ (0xb58） |  |  |  |  |  |  |  |  |
| Barometric pressure sensor characteristics UUID | 0x1627 |  |  |  |  |  |  |  |  |
| Sensor data permission | Read and Notification |  |  |  |  |  |  |  |  |
| Sensor data bytes | 4 bytes (Little-Endian） |  |  |  |  |  |  |  |  |
| Sensor pressure data value: 32-bit unsigned integer（1 pa） | 1st byte (MSB) |  |  |  | 2nd byte |  | 3rd byte |  | 4th byte (LSB) |
|  | (LSB) |  |  |  |  |  |  |  | (MSB) |
| Example | 0x7f8a0100 indicates that the current barometric pressure is 100991pa (0x18a7f ) |  |  |  |  |  |  |  |  |
| Air extraction cover signal characteristics UUID | 0x1628 |  |  |  |  |  |  |  |  |
| Permission to report data | Read and Notification |  |  |  |  |  |  |  |  |
| Reported data bytes | 1 byte |  |  |  |  |  |  |  |  |
| Reported data value | 0x00 | Air extraction cover open |  |  |  |  |  |  |  |
|  | 0x01 | Air extraction cover close |  |  |  |  |  |  |  |
| Example | 0x01 indicates that Air extraction cover close currently |  |  |  |  |  |  |  |  |
| solenoid valve control characteristics UUID | 0x1629 |  |  |  |  |  |  |  |  |
| Start data permission | Write |  |  |  |  |  |  |  |  |
| Start data byte | 1 byte |  |  |  |  |  |  |  |  |
|  | 0x01 | solenoid valve open (non - 0 data) |  |  |  |  |  |  |  |
|  | 0x00 | solenoid valve Stop |  |  |  |  |  |  |  |
| Example | 0x01 indicate that solenoid valve open |  |  |  |  |  |  |  |  |
| Infrared Remote Control characteristics UUID | 0x162A |  |  |  |  |  |  |  |  |
| Start data permission | Write |  |  |  |  |  |  |  |  |
| Start data byte | 1 byte |  |  |  |  |  |  |  |  |
|  | 0x01 — Increase the brightness of the flashlight （Equivalent to short pressing the left button on the remote control） 0x02 — Switch the type of flashlight light （Equivalent to short pressing the middle button of the remote control） 0x03 — Reduce the brightness of the flashlight （Equivalent to short pressing the right button on the remote control） 0x04 — Flashlight enters sleep mode （Equivalent to long pressing the middle button of the remote control） 0x05 — The flashlight is starting to wake up （Equivalent to long pressing the middle button of the remote control again） 0x06 — Flashlight enters focus or flashing mode (Equivalent to simultaneously pressing the left and right buttons on the remote control) |  |  |  |  |  |  |  |  |
| Example | 0x01 The current LED brightness level of the flashlight has been increased by 1 level |  |  |  |  |  |  |  |  |

Table 8